2018

College of Engineering

Department of Computer Engineering and Computer Science

UART Full Duplex Protocol and Technology Specific Instantiation Chip Specification

### PREPARED FOR

Mr. John Tramel

CECS 460 System-On-Chip Design

### PREPARED BY

Jose Sotelo

Table of Contents

1 Introduction5

1.1	Purpose5

2 Applicable Documents 6

2.1	External Documents6

2.2	PicoBlaze 8-bit Microcontroller User’s Guide……………………………………6

2.3	TramelBlaze…………………………………………………………………….6-7

2.4	TramelBlaze ISA………………………………………………………………….8

3 Requirements9

3.1.1	Performance Requirements9

3.1.2	Interface Requirements9

3.1.3	Physical Requirements9

3.1.4	Power Requirements…………………………………………………………...9-10

4 Implementation11

4.1.1	Design Description………………………………………………………...…11-12

4.1.2	Top Level Description12

4.1.3	Block Diagram13

4.1.4	Input/Output Interface Description………………………………………………14

4.1.5	Clocks14

4.1.6	Resets14

5 Internally Acquired Blocks15

5.1	Core_Logic………………………………………………………………………15

5.2 	TSI……………………………………………………………………………….16

5.3 	UART_Top…………………………………………………………………...17-19

5.4	Tx Engine…………………………………………………………………….20-21

5.5	Rx Engie……………………………………………………………………...22-25

5.6 	Rx FSM………………………………………………………………………….26

5.7	Rx Remap………………………………………………………………………..27

5.8	Baud Decoder…………………………………………………………………….28

5.9	Tx Shift Register…………………………………………………………………29

5.10	Tx Bit Time Counter……………………………………………………………..30

5.11	Tx Bit Counter…………………………………………………………………...31

5.12	Tx Parity Gen…………………………………………………………………….32

5.13	Rx Bit Time Counter……..………………………………………………………33

5.14	Rx Bit Counter…………………………………………………………………...34

5.15	SR Flop…………………………………………………………………………..35

5.16	PED………………………………………………………………………………36

5.17	AISO……………………………………………………………………………..37

6 Externally Acquired Blocks38

6.1	TramelBlaze Embedded Processor38

6.2	Description38

6.3	Block Diagram38

6.4	I/O Definition38-39

6.5	I/O Timing……………………………………………………………………….39

7 Chip Verification40

7.1	Testbench WaveForms40-43

8 Software Code44

Top_Level.v…………………………………………………………………..44-45

Core_Logic…………………………………………………………………...46-48

TSI.v………………………………………………………………………….49-51

AISO.v…………………………………………………………………………...52

UART_Top.v…………………………………………………………………53-55

SR_Flop.v………………………………………………………………………..56

Baud_Dec.v………………………………………………………………………57

Tx_Engine.v………………………………………………………………….58-60

Rx_Engine.v………………………………………………………………….61-62

PED………………………………………………………………………………63

SR_Flop_Txrdy.v………………………………………………………………..64

Tx_Load_Reg.v………………………………………………………………….65

Tx_Parity_Gen_Dec……………………………………………………………..66

Tx_shift_register.v……………………………………………………………….67

Tx_bit_time_counter.v………………………………….………………………..68

Tx_bit_counter.v…………………………………………………………………69

Rx_Engine_Control.v………………………………………………………...70-71

Rx_FSM.v……………………………………………………………………72-73

Rx_bit_time_counter.v……………………………………………………..……74

Rx_bit_counter.v…………………………………………………………..…74-76

Rx_DataPath.v………………………………………………………………..77-79

Rx_Shift_Reg.v…………………………………………………………………..80

Rx_Remap.v……………………………………………………………………...81

Assembly Code……………………………………………………………….82-93

Appendix A: Key Terms 94

1.	Introduction

The Universal Asynchronous Receiver Transmitter (UART) is a very common useful full duplex serial communication protocol. It serves as a basis for many ubiquitous protocols such as RS-232 (COM ports in computers). Its basis is a two-wire communication on which one port transmits and the other receives. The basic data units transferred is 1-byte.  Data is transferred from the data bus to the transmitting UART in parallel from. Data that is been transmitted is organized into packets. Each packet contains 1 start bit, 5-9 data bits, an optional parity bit and 1 or 2 stop bits. The transferring speed of the data depends on the baud rate. The baud rate is a unit of measurement of bits per second (bps). Once the baud rate is selected, the data packet will be transferred to the receiving pin.

During the transmitting stage, the UART gets the parallel data from the data bus, it adds a start bit, a parity bit, and a stop bit. The data put together is output serially, bit by bit at the Tx pin. The receiving UART read the data bit by bit at its Rx pin. Once the data has been received, it’s then converted back into parallel form and removes the start bit, parity but and stop bits. Lastly, the receiving UART transfers the data back in parallel to the data bus on the receiving end.

In Revision 2, we interface the full duplex UART circuit with the technology specific instantiation (TSI) circuit. The TSI contains all references to the target technology library. All the communications between the inputs and outputs of the device pass through the TSI. The I/O cells for a design are one of the primary inclusions of the TIS block. Each I/O of the chip must have a particularly selected device to meet electrical and timing requirements of the external interface. We will be using Nexys Artix-7 Library Guide which defines the cells and their capabilities.

1.1	Purpose

For this project, we will be designing an SOPC (System on Programmable Chip) that contains the Transmit Engine, Receive Engine and technology specific instantiation (TSI). The two engines created in this chip specification will make the full duplex UART protocol. The user will be able to configure the transfer of 8 or 7 bits, a parity bit, odd or even bit and the baud rate. We will interface the Transmit and Receive Engine with the 16-bit TramelBlaze and the TSI to send and receive data via a serial terminal RealTerm. The serial terminal will output a Banner, prompt the user, display the hometown of the designer, delete and input characters, and print the number of characters in the current line.

2.	Applicable Documents:

2.1	External Documents

These documents were used in the development of the full duplex UART protocol. There main purpose was to assist the designer in the understanding of the embedded microcontroller the TramelBlaze and how to interface it with other FPGA logic.

2.2 PicoBlaze 8-bit Microcontroller User’s Guide

The PicoBlazeTM embedded microcontroller is an efficient, cost-effective embedded processor core for Spartan -3, Virtex -II, and Virtex-II Pro PFGAs. It is natively hosted on the Nexys 3/6 families. The 8-bit PicoBlaze microcontroller is specially designed and optimized for the Spartan -3, Virtex -II, and Virtex-II Pro architectures thus allowing for other FPGA logic to be connect to an embedded microcontroller’s input and output ports.

2.3 TramelBlaze

The TrambelBlaze is a 16-bit processor core designed to emulate the Xilinx PicoBlaze. We will be using the TramelBlaze to interface it with develop FPGA logic. Since we switch on to the Nexys 4 FPGA, the PicoBlaze is no longer supported. Thus, the TramelBlaze was developed by John Tramel to allow the student to interface the embedded microcontroller with their own developed FPGA logic. The instruction set for the TramelBlaze is the PicoBlaze instruction set.

The TramelBlaze has the following:

- 16 16-bit wide general-purpose registers (r0-rF)

- 4096 Instruction Program Store - The code ROM provides storage for a suitable number of instructions. The instructions are either one or two words each.

- Arithmetic Logic Unit (ALU) – 16-bit wide ALU performs all microcontroller operations such as basic arithmetic, bitwise logic operations, arithmetic compare, comprehensive shift and rotate operations, program control operation such as jump or call.

- Flags – The Zero flag indicates the result of the previous operation was zero and the CARRY flag indicates various conditions. The INTERRUPT_ENABLE flag indicates interrupt operations.

- 512-word scratchpad – A RAM within the processor with access provided by STORE and FETCH instructions. STORE copies the contents of a register to the RAM and FETCH copies the contents of the RAM to a register.

- I/O – the Input/Output ports extends the TramelBlaze’s capabilities to allow interfacing to other PFGA logic. The addressing capabilities of the I/O instructions is 0 to 65535 (FFFF) which will require an external address decoder. INPUT operations move data from surrounding FPGA logic into a register and the OUTPUT operations move data from a register to the surrounding FPGA logic.

- Program Counter – The PC points to the next instruction to be executed. JUMP, CALL, RETURN, RETURNI instructions and interrupt or rest events will modify the contents of the PC.

2.4 Programming TramelBlaze

3.	Requirements:

3.1.1	Performance Requirements

The full duplex Universal Asynchronous Receiver will be designed in the Nexy4 DDR FPGA board. It will be able to transmit and receive data via the following baud rates: 300, 1200, 2400, 4800, 9600, 19200, 38400, 57600, 115200, 230400, 460800 and 921600.

3.1.2	Interface Requirements

The Transmit Engine, Receive Engine, TramelBlaze, TSI, and the serial terminal RealTerm will be used to transmit and receive data.

3.1.3	Physical Requirements

The highest baud rate the full duplex UART engine is capable to communicate via the serial terminal without breaking the data being sent is 921600.

3.1.4	Power Requirements

The Nexys4 DDR board can receive power from the Digilent USB-JTAG port(J6) or from an external power supply. Jumper JP3 (near the power jack) determines which source is used. All Nexys4 DDR power supplies can be turned on and off by a single logic-level power switch (SW16).

A power-good LED (LD22), driven by the “power good” output of the ADP2118 supply, indicates that the supplies are turned on and operating normally.

The USB port can deliver enough power for the vast majority of designs. The out-of-the-box demo draws ~400mA of current from the 5V input rail. Depending on your application, more power might be required to power your design. Some applications can be run without being connected to a PC’s USB port. In these instances, an external power supply or battery pack can be used.

External power supply can be used by plugging into the power jack (JP3) and setting jumper J13 to “wall”. The supply must use a coax, center-positive 2.1mm internal-diameter plug and deliver 4.5VDC to 5.5VDC and at least 1A of current.

External battery pack can be used by connecting the battery’s positive terminal to the center pin of JP3 and the negative terminal to the pin labeled J12, directly below JP3. Since the main regulator on the Nexys4 DDR cannot accommodate input voltages over 5.5VDC, an external battery pack must be limited to 5.5VDC. If the USB Host function (J5) is used, at least 4.6V needs to be provided. In other cases, the minimum voltage is 3.6V.

4.	Implementation

4.1.1	Design Description

This project implements the Transmit and the Receive Engine with selectable baud rates, the number of bits transmitted, parity enable/disable and odd/even parity bits. For the Transmit Engine to function, the designer needs to implement SR flip flops, an 8-bit loadable register, one-bit register, the parity bit decoder, bit time counter, bit counter and shift register. For the Receive Engine, the designer must implement receive engine control and data path. The goal of the receive engine is to synchronize the data collection with the Tx communication by always polling the Rx line looking for a high to low transition indicating the arrival of the Start bit. A simple finite state machine ensures that the Start bit remains active until the mid-bit at which time the data collection will proceed at a bit time until all bits are received. The FSM will output Start and DoIt indicating that the receive engine is looking for a Start bit and that the receive engine is currently processing data.

The receive engine data path is used to read and write data. There are two addresses dedicated to the UART: 0000 (UART DATA) and (UART Status). When the TramelBlaze writes address 0000, the transmit engine will receive the data byte. When the TramelBlaze reads address 0000 the receive engine will respond with the received data. When the TramelBlaze reads address 0001 it will receive the status collected by the receive engine along with two ready bits (tx ready, rx ready). The other three status flags PERR (Parity Error) indicates that a Parity Error has been detected, FERR (Framing Error) n indicates that the synchronization is lost when no Stop bit is found, and lastly OVF (Overflow) indicates that another byte is being received before the TramelBlaze has read the previous byte.

| Eight | Parity Enable | Odd and Even | Bit 10 | Bit 9 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | EP |
| 0 | 1 | 1 | 1 | OP |
| 1 | 0 | 0 | 1 | D7 |
| 1 | 0 | 1 | 1 | D7 |
| 1 | 1 | 0 | EP | D7 |
| 1 | 1 | 1 | OP | D7 |

The first block inside the Tx Engine, is a SR flip flop that is used to set the TxRdy signal high at reset. The second SR flop goes low at reset and both SR flops are used to maintain stable outputs after the inputs are turned off. The 8-bit loadable register that is used to load the data in when the load input pin goes high and outputs 8 bits of data. The 8 bits of data are then used to determine how many bits are needed to produce a transmission of data. The user determines what type of protocol to send out. The are 7 options to transmit data from the inputs eight, pen (parity enable), and ohel (odd and even bits): 7N1, 7E1, 7O1, 8N1, 8E1, and 8O1. The truth table is used to produce the combo logic circuit that produces the output bits 10 and 9.

At the heart of the Transmit Engine, is a 11-bit shift register that is used to shift the transmitted data one bit of a time. Once the parity decoder determines bit 10 and bit 9, a one-bit register goes high, it tells the shift register to load the data to bit-10, bit-9, bits 8-2 and bit-1 goes high and bit-0 goes low. If the reset is pressed, the shift registers outputs 11-bits 1’s. The output of the shift register gets the LSB 0 of the data being shifted.

The design must also include two counters: 1) Count clocks to determine the bit time and 2) Count the bits to know when we are done. These two counters will let us know when we have waited a bit time and to know when all the bits have been transmitted. Lastly, a Baud Decoder circuit is created to generate the number of clocks that are necessary to fill one-bit time. Combining all the modules describe above, produces the Transmit Engine. Once the Transmit and Receive Engine are completed, you can interface the UART with the 16-bit TramelBlaze microcontroller. For the user to select a different baud speed, the following table is used to select the baud by using the on-board switches.

| Baud Switches | Baud Rate | Bit Time | Nexys4 Count |
| --- | --- | --- | --- |
| 0000 | 300 | 0.003333 | 333,333 |
| 0001 | 1200 | 0.0008333 | 83,333 |
| 0010 | 2400 | 0.000416667 | 41,667 |
| 0011 | 4800 | 0.000208333 | 20,833 |
| 0100 | 9600 | 0.000104167 | 10,417 |
| 0101 | 19200 | 5.20833E-05 | 5,208 |
| 0110 | 38400 | 2.60417E-05 | 2,604 |
| 0111 | 57600 | 1.73611E-05 | 1,736 |
| 1000 | 115200 | 8.68056E-05 | 868 |
| 1001 | 230400 | 4.34028E-06 | 434 |
| 1010 | 460800 | 2.17014E-06 | 217 |
| 1011 | 921600 | 1.08507E-06 | 109 |

4.1.2	Top Level Description

The top-level block diagram contains the Core design and the TSI. The Core design consist of the full UART and TSI contains all the references to the target technology libraries. For the Core design to communicate with the outside world, it’s I/O’s must pass through the TSI before interacting with the FPGA. The assembly code written for the TramelBlaze will be used to display text to the serial terminal. The requirements are to display a banner, hometown when entering an asterisk, line count, and backspace to delete previously entered characters.

4.1.3 Top Level Block Diagram

| I/O |  | Signal Description | FPGA board Switches & Buttons |
| --- | --- | --- | --- |
| Input | clk | 100MHz FPGA board crystal oscillator | None |
| Input | rst | Synchronous Rest |  |
| Input | baud | 4-bit input used to select baud rate | LOC Pin Assignment: R13(SW8), U18(SW7), T18(SW6), R17(SW5) |
| Input | eight | 1-bit input to select [7:0] ot [6:0] bits | LOC Pin Assignment: R15(SW4) |
| Input | p en | Parity enable – 1: odd parity, 0: no parity | LOC Pin Assignment: M13(SW3) |
| Input | ohel | odd high even low- 1: odd parity, 0: even parity | LOC Pin Assignment: L16(SW2) |
| Input | rx | Receive signal which indicates a high to low transition | LOC Pin Assignment: C4 |
| Output | tx | Transmission signal which indicates that data is being shifted out from Transmit Engine | LOC Pin Assignment: D4 |

4.1.5	Clocks

All blocks in the design used one clock that runs at a frequency of 100MHz and period of 10 nanoseconds.

4.1.6	Resets

All the designs used a synchronous reset (AISO) to prevent metastability. The purpose for the AISO circuit in our design is to prevent any of the flops in our design from entering a metastable state when reset is released. Synchronously releasing reset ensures all-of the flops are reset at the same time and that they are stable when reset is released.

5. Internally Acquired Blocks

5.1	Core Logic

The Core Logic Circuit is used to combine the Address Decoder, UART, and TramelBlaze so it could interface with the technology specific instantiation (TSI) circuit. The core logic also contains and AISO, PED, SR flop, counters, load register, and shift register that are essential to the design.

| Name | I/O | Description |
| --- | --- | --- |
| clk | I | Synchronous clock |
| rst | I | Synchronous reset |
| baud[3:0] | I | 4-bit input to select baud rate |
| eight | I | 1-bit input to select 8-bit data transmission |
| ohel | I | 1-bit input to select if we want even or odd |
| p_en | I | 1-bit input to select parity bit |
| rx | I | Receive signal looks for a high to low transition indicating the arrival of the start bit |
| leds | O | LEDs are used to demonstrate the TramelBlaze is reading data |
| tx | O | Data being shifted out of the transmit engine |

5.2	Technology Specific Instantiation (TSI)

The TSI contains all references to the target technology library. All communications to or from the I/O of the device pass through the TSI. Each I/O of the chip must have a particular selected device to meet electrical timing requirements of the external interface.

| Name | I/O | Description |
| --- | --- | --- |
| i_baud[3:0] | I | Baud input buffer |
| i_leds | I | LEDs input buffer |
| i_clk | I | Clock input buffer |
| i_eight | I | Eight input buffer |
| i_ohel | I | Odd high even low input buffer |
| i_p_en | I | Parity enable input buffer |
| i_rst | I | Reset input buffer |
| i_rx | I | Receive input buffer |
| i_tx | I | Transmit input buffer |
| o_baud[3:0] | O | Baud output buffer |
| o_leds | O | LEDS output buffer |
| o_clk | O | Clock output buffer |
| o_eight | O | Eight output buffer |
| o_ohel | O | Odd high even low output buffer |
| o_p_en | O | Parity enable buffer |
| o_rst | O | Rest output buffer |
| o_rx | O | Receive output buffer |
| o_tx | O | Transmit output buffer |

5.3	UART Top

The UART Top in composed of the Transmit Engine, Receive Engine, Baud Decoder, UART status flags and pulse edge detectors. Both engines in this design require a specific baud rate selected by the user. An interrupt signal is also implemented to cause the TramelBlaze to enter an Interrupt Service Routine. The UART top also contains status flag such as parity error, over flow, and framing error. The tx signal is used to transmit data over USB.

| Name | Signal Description | I/O | FPGA board Switches & Buttons |
| --- | --- | --- | --- |
| clk | 100MHz FPGA board crystal oscillator | I | None |
| rst | Synchronous Rest | I | LOC Pin Assignment: M18 |
| write | Write address 0000 | I | None |
| read | Read address 0001 | I | None |
| out_port | 8-bit data coming from TramelBlaze | I | None |
| baud | 4-bit input used to select baud rate | I | LOC Pin Assignment: R13(SW8), U18(SW7), T18(SW6), R17(SW5) |
| eight | 1-bit input to select [7:0] ot [6:0] bits | I | LOC Pin Assignment: R15(SW4) |
| p_en | Parity enable – 1: odd parity, 0: no parity | I | LOC Pin Assignment: M13(SW3) |
| ohel | odd high even low- 1: odd parity, 0: even parity | I | LOC Pin Assignment: L16(SW2) |
| rx | Receive signal which indicates a high to low transition | I | LOC Pin Assignment: C4 |
| tx | Transmission signal which indicates that data is being shifted out from Transmit Engine | O | LOC Pin Assignment: D4 |
| UART_DS | Status flags: parity error, framing error, and over flow error | O | None |
| UART_Int | Set an interrupt signal that gets feed into the TramelBlaze | O | None |

5.4	Transmit Engine

The Transmit Engine is composed of an SR flop, loadable register, parity decoder, a register, shift register, bit time counter, and a bit counter.

| Name | I/O | Description |
| --- | --- | --- |
| baud_count[18:0] | I | Baud Rate Decoder |
| data_in[7:0] | I | Data from TramelBlaze |
| clk | I | 100MHz crystal oscillator |
| eight | I | 8 bits |
| ohel | I | Odd high even low |
| p_en | I | Parity enable |
| load | I | From Address Decoder |
| rst | I | AISO |
| tx | O | USB |
| tx rdy | O | UART Interrupt |

5.5	Receive Engine

The Receive Engine is composed of control and data path. The control path contains a finite state machine, bit time counter and bit counter to sync the signals coming into the UART. The data path checks for the correct transmission, if the transmission is incorrect the status flags are set to indicate which type of error it was. Otherwise, the data will be outputted to the TramelBlaze.

| Name | I/O | Description |
| --- | --- | --- |
| k[18:0] | I | Baud Rate Decoder |
| clk | I | 100MHz crystal oscillator |
| eight | I | On board switch |
| ohel | I | On board switch |
| p en | I | On board switch |
| read | I | TramelBlaze |
| rst | I | AISO |
| rx | I | UART Interrupt |
| data to TB[7:0] | O | TramelBlaze |
| frame err | O | Framing error |
| over flow | O | Overflow error |
| parity err | O | Parity error |
| rx rdy | O | TramelBlaze |

5.6	Rx FSM

The Receive Engine contains a finite state machine that determines if its receiving a signal or not.

| Name | I/O | Description |
| --- | --- | --- |
| btu | I | Rx bit time counter |
| clk | I | 100MHz crystal oscillator |
| done | I | Rx bit counter |
| rst | I | AISO |
| rx | I | USB |
| do it | I | Rx bit time counter and bit counter |
| start | I | Rx bit time counter |

5.7	Rx_Remap

The purpose for the Rx ramap circuit is to justify the received data. Depending on the transmission data, the remap will adjust the data according to the eight and parity enable select switches. 8-bits from the remap circuit will go to the TramelBlaze, while the remaining bits will be distributed accordingly.

| Name | I/O | Description |
| --- | --- | --- |
| data in[9:0] | I | Data coming in from shift register |
| sel[1:0] | I | Eight and Parity enable inputs |
| remap[9:0] | O | Data out |

5.8	Baud Decoder

The Baud decoder is a mux that selects how many bits per second to run the design.

| Name | I/O | Description |
| --- | --- | --- |
| baud[3:0] | I | On board switches |
| baud count | O | Transmit Engine |

5.9	Tx Shift Register

The shift register is responsible for taking in the data and shifting them into a register. If the switches eight and parity enable are selected, the data must be remapped accordingly.

| Name | I/O | Description |
| --- | --- | --- |
| clk | I | 100MHz crystal oscillator |
| rst | I | AISO |
| load | I | Load |
| shift | I | Shift Data |
| data in[10:0] | I | Data coming in |
| sdo | O | Data shifted out |

5.10	Tx Bit Time Counter

The Bit Time Counter is used to determine the speed at which the engine runs caused by the value selected in the baud decoder.

| Name | I/O | Description |
| --- | --- | --- |
| baud[3:0] | I | On board switches |
| sel[1:0] | I | DoIt and BTU |
| clk | I | 100MHz crystal oscillator |
| btu | O | Tx shift register and bit count |

5.11	Tx Bit Time Counter

The Bit Counter is used to count the number of bits to ensure that the engine is picking up the correct number of bits. It also checks the values to see if there is any discrepancies by seen how many bits are need to receive based on the eight and parity enable switches.

| Name | I/O | Description |
| --- | --- | --- |
| sel[1:0] | I | DoIt and BTU |
| clk | I | 100MHz crystal oscillator |
| rst | I | AISO |

5.12	Tx Parity Generator Decoder

The transmit parity generator decoder is responsible for generating the even or odd parity bits depending on the configuration. If eight is selected, the loadable register will load 8 bits, else 7-bits. Depending on the data transmitted, if the data transmitted requires a parity bit the data will be account for and an even or odd no parity bit will be added.

| Name | I/O | Description |
| --- | --- | --- |
| load data[7:0] | I | 8 bit data coming loadable register |
| eight | I | On board switch |
| ohel | I | On board switch |
| p_en | I | On board switch |
| bit9 | O | 9-bit data |
| bit10 | O | 10-bit data |

5.13	Rx Bit Time Counter

The bit timer counter is responsible for counting the number of clocks depending on the baud rate that is selected. The baud rate is configurable with the on-board switches. The baud rate determines the bit time which is the length of time that a transmitted bit is held on the wire.

| Name | I/O | Description |
| --- | --- | --- |
| k | I | Baud rate decoder |
| sel[1:0] | I | DoIt and Btu |
| clk | I | 100MHz clock oscillator |
| rst | I | AISO |
| start | I | FSM |
| btu | O | register |

5.14	Rx Bit Counter

The bit counter in the transmit engine is responsible for counting the number of bits that have been transmitted. The purpose of this block is to count the number of bits that have been transmitted. The number of bits transmitted is always 11 and once all 11-bits have been transmitted, the done signal goes high which set the tx_rdy signal high which indicates that the transmit engine is ready to transmit another byte of data.

| Name | I/O | Description |
| --- | --- | --- |
| sel[1:0] | I | DoIt and Btu |
| clk | I | 100MHz clock oscillator |
| eight | I | On board switches |
| p en | I | On board switches |
| rst | I | AISO |
| done | O | FSM and Rx Controller |

5.15	SR Flop

The SR flop are used to store information after the inputs are turned off. The set input causes the output of 1 and the R input cause the output of 0. For the transmit engine, when we reset the design, we need to set the reset value high.

| Name | I/O | Description |
| --- | --- | --- |
| clk | I | 100MHz crystal oscillator |
| R | I | TramelBlaze |
| rst | I | AISO |
| S | I | PED |
| Q | O | TramelBlaze |

5.13	Pulse Edge Detector (PED)

The PED is used to generate a one clock period pulse whenever the signal is asserted.

| Name | I/O | Description |
| --- | --- | --- |
| clk | I | 100MHz crystal oscillator |
| ped in | I | UART |
| rst | I | AISO |
| ped out | O | SR flop |

5.14	Asynchronous-In-Synchronous-Out (AISO)

The Asynchronous-In-Synchronous-Out circuit is used to synchronize the reset of digital flip flops.

| Name | I/O | Description |
| --- | --- | --- |
| clk | I | 100MHz crystal oscillator |
| rst | I | Button |
| rst s | O | Output to every block |

6.	Externally Acquired Blocks

6.1	TramelBlaze

The TramelBlaze is a 16-bit embedded microcontroller designed to emulate the 8-bit PicoBlaze.

6.2	Description

The TramelBlaze is a 16-bit processor core designed to emulate the Xilinx PicoBlaze. The instruction set is the PicoBlaze instruction set. It contains three memories Instructions Rom, Call/Return Stack and Scratchpad RAM. The TramelBlaze’s I/O are a 16-bit in_Port, Interrupt, Reset, Clk, a 16-bit Out_Port, 16-bit Port_Id output, Read_Strobe output, Write_Strobe output and interrupt acknowledge output. When implementing the TramelBlaze into your design, you must generate the tb_rom and load the coe file into RAM. A Python assembler is used to generate assembly code. We will be using assembly code to output to the serial terminal and keep track of the line count.

6.3	TramelBlaze Block Diagram

6.4	I/O Definitions

The I/O ports extend the TramelBlaze MCU’s capabilities and allow the MCU to connect custom peripherals or to FPGA. It contains a 16-bit in_Port input that is used to write to the scratchpad memory. It allows to use 16 16-byte-wide registers. It also handles the interrupt input used to let the MCU that an event occurred and executes interrupt service routine corresponding to the received interrupt. Once the interrupt is complete, it returns to the main program. The TramelBlaze shares the 100MHz clock from the FGPA. The 16-bit output Port_ID produces the address/instruction. The 16-bit Out_Port produces data contained within the TramelBlaze. Read_Strobe output is asserted high indicating that the input data on the in_Port was captured to the specified data register during an input instruction. The Write_Strobe is asserted high validating the output signal data of the Out_Port. Lastly, the Int_Ack is asserted high to acknowledge that an interrupt event occurred.

6.5	I/O Timing

\

7.	Chip Verification

7.1 TestbenchWaveforms

Figure 27: Transmit Engine TestFixture

Figure 28: Shift Register TestFixture

Figure 29: Parity Decoder TestFixture

To the left truth table Inputs: Data_In, Eight, Parity Enable and Ohel. Outputs: bit10 and bit 9 on the right.

Figure 30: Receive Engine TestFixture

Figure 31: Receive Engine Control Path TestFixture

Figure 32: Receive Engine Finite State Machine TestFixture

Figure 33: Receive Engine Remap TestFixture

Figure 34: Receive Engine Shift Register TestFixture

Figure 35: Full UART and TSI RealTerm Output

Figure 36: Transmit Engine RealTerm Output

8.	Software Code

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Final Project>                           //

// File name:  Top_Level.v                               //

//                                                       //

// Created by <Jose Sotelo> on <April 16, 2018>          //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Top_Level(clk, rst, baud, eight, p_en, ohel, rx, tx, leds);

input               clk, rst;

input     [3:0]     baud;

input               eight;

input               p_en;

input               ohel;

input               rx;

output              tx;

output    [15:0]    leds;

wire                w_clk, w_rst;

wire      [3:0]     w_baud;

wire                w_eight;

wire                w_p_en;

wire                w_ohel;

wire                w_rx;

wire      [15:0]    w_leds;

//***********************************

//   Core Logic

//***********************************

Core_Logic

uart_core(

.clk(w_clk),

.rst(w_rst),

.baud(w_baud),

.eight(w_eight),

.p_en(w_p_en),

.ohel(w_ohel),

.rx(w_rx),

.tx(w_tx),

.leds(w_leds)

);

//***********************************

//   TSI

//***********************************

TSI

tsi_buf(

.i_clk(clk),

.i_rst(rst),

.i_baud(baud),

.i_eight(eight),

.i_p_en(p_en),

.i_ohel(ohel),

.i_rx(rx),

.i_tx(w_tx),

.i_leds(w_leds),

.o_clk(w_clk),

.o_rst(w_rst),

.o_baud(w_baud),

.o_eight(w_eight),

.o_p_en(w_p_en),

.o_ohel(w_ohel),

.o_rx(w_rx),

.o_tx(tx),

.o_leds(leds)

);

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Final Project>                           //

// File name:  Core_Logic.v                              //

//                                                       //

// Created by <Jose Sotelo> on <April 16, 2018>          //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Core_Logic(clk, rst, baud, eight, p_en, ohel, rx, tx, leds);

input               clk, rst;

input     [3:0]     baud;

input               eight, p_en, ohel;

input               rx;

output              tx;

output    [15:0]    leds;

reg       [15:0]    leds;

wire                w_rst_s;

wire                w_write_strobe;

wire                w_read_strobe;

wire                w_int_ack;

wire      [7:0]     w_uart_ds;

wire                w_uart_int;

wire                w_interrupt;

wire      [15:0]    w_port_id;

wire      [15:0]    w_out_port;

reg       [15:0]    w_write;

reg       [15:0]    w_read;

//***********************************

//   AISO

//***********************************

Aiso

aiso(

.clk(clk),

.rst(rst),

.rst_s(w_rst_s)

);

//***********************************

//   Address Decode

//***********************************

always @(*)

begin

w_write = 16'b0;

w_read  = 16'b0;

w_write[w_port_id] = w_write_strobe;

w_read[w_port_id]  = w_read_strobe;

end

//***********************************

//   UART

//***********************************

UART_Top

uart(

.clk(clk),

.rst(w_rst_s),

.write(w_write[0]),

.read(w_read[1:0]),

.out_port(w_out_port[7:0]),

.rx(rx),

.baud(baud),

.eight(eight),

.p_en(p_en),

.ohel(ohel),

.tx(tx),

.UART_DS(w_uart_ds),

.UART_INT(w_uart_int)

);

//***********************************

//   SR Flop Interrupt

//***********************************

SR_Flop

sr_interrupt(

.clk(clk),

.rst(w_rst_s),

.S(w_uart_int),

.R(w_int_ack),

.Q(w_interrupt)

);

//***********************************

//   TramelBlaze

//***********************************

tramelblaze_top

tb_top(

.CLK(clk),

.RESET(w_rst_s),

.IN_PORT({8'b0, w_uart_ds}),

.INTERRUPT(w_interrupt),

.OUT_PORT(w_out_port),

.PORT_ID(w_port_id),

.READ_STROBE(w_read_strobe),

.WRITE_STROBE(w_write_strobe),

.INTERRUPT_ACK(w_int_ack)

);

//***********************************

//   Walking LEDs

//***********************************

always @(posedge clk, posedge w_rst_s)

begin

if(w_rst_s)

leds <= 16'b0;

else if(w_write[2])

leds <= w_out_port;

else

leds <= leds;

end

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Final Project>                           //

// File name:  TSI.v                                     //

//                                                       //

// Created by <Jose Sotelo> on <April 16, 2018>          //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module TSI(i_clk, i_rst, i_baud, i_eight, i_p_en, i_ohel, i_rx, i_tx, i_leds,

o_clk, o_rst, o_baud, o_eight, o_p_en, o_ohel, o_rx, o_tx, o_leds);

input               i_clk;

input               i_rst;

input     [3:0]     i_baud;

input               i_eight;

input               i_p_en;

input               i_ohel;

input               i_rx;

input               i_tx;

input     [15:0]    i_leds;

output              o_clk;

output              o_rst;

output    [3:0]     o_baud;

output              o_eight;

output              o_p_en;

output              o_ohel;

output              o_rx;

output              o_tx;

output    [15:0]    o_leds;

// BUFG: Global Clock Simple Buffer

// 7 Series

// Xilinx HDL Libraries Guide, version 2012.2

BUFG

BUFG_inst (

.O(o_clk), // 1-bit output: Clock output

.I(i_clk)  // 1-bit input: Clock input

);

// IOBUF: Single-ended Bi-directional Buffer

// All devices

// Xilinx HDL Libraries Guide, version 2012.2

IBUF #(

.IBUF_LOW_PWR("TRUE"), // Low Power - "TRUE", High Perfor = "FALSE"

.IOSTANDARD("DEFAULT") // Specify the I/O standard

) rst(

.O(o_rst), // Buffer output

.I(i_rst)  // Buffer input

);

IBUF #(

.IBUF_LOW_PWR("TRUE"), // Low Power - "TRUE", High Perfor = "FALSE"

.IOSTANDARD("DEFAULT") // Specify the I/O standard

) baud[3:0](

.O(o_baud), // Buffer output

.I(i_baud)  // Buffer input

);

IBUF #(

.IBUF_LOW_PWR("TRUE"), // Low Power - "TRUE", High Perfor = "FALSE"

.IOSTANDARD("DEFAULT") // Specify the I/O standard

) eight(

.O(o_eight), // Buffer output

.I(i_eight)  // Buffer input

);

IBUF #(

.IBUF_LOW_PWR("TRUE"), // Low Power - "TRUE", High Perfor = "FALSE"

.IOSTANDARD("DEFAULT") // Specify the I/O standard

) p_en(

.O(o_p_en), // Buffer output

.I(i_p_en)  // Buffer input

);

IBUF #(

.IBUF_LOW_PWR("TRUE"), // Low Power - "TRUE", High Perfor = "FALSE"

.IOSTANDARD("DEFAULT") // Specify the I/O standard

) ohel(

.O(o_ohel), // Buffer output

.I(i_ohel)  // Buffer input

);

IBUF #(

.IBUF_LOW_PWR("TRUE"), // Low Power - "TRUE", High Perfor = "FALSE"

.IOSTANDARD("DEFAULT") // Specify the I/O standard

) rx(

.O(o_rx),  // Buffer output

.I(i_rx)   // Buffer input

);

// OBUF: Single-ended Output Buffer

// 7 Series

// Xilinx HDL Libraries Guide, version 2012.2

OBUF #(

.DRIVE(12),             // Specify the output drive strength

.IOSTANDARD("DEFAULT"), // Specify the output I/O standard

.SLEW("SLOW")           // Specify the output slew rate

)tx (

.O(o_tx),  // Buffer output (connect directly to top-level port)

.I(i_tx)   // Buffer input

);

OBUF #(

.DRIVE(12),             // Specify the output drive strength

.IOSTANDARD("DEFAULT"), // Specify the output I/O standard

.SLEW("SLOW")           // Specify the output slew rate

)leds[15:0] (

.O(o_leds),  // Buffer output (connect directly to top-level port)

.I(i_leds)   // Buffer input

);

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Aiso.v                                    //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Aiso(clk, rst, rst_s);

input     clk;

input     rst;

output    rst_s;

reg       q1, q2;

always @(posedge clk, posedge rst)

if(rst)

{q1,q2} <= 2'b0;

else

{q1,q2} <= {1'b1, q1};

//***********************************

//   Assign Statement

//   Modeling combinational logic

//***********************************

assign rst_s = ~q2;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  UART_Top.v                                //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module UART_Top(clk, rst, write, read, out_port, rx, baud,

eight, p_en, ohel, tx, UART_DS, UART_INT);

input               clk, rst;

input               write;

input     [1:0]     read;

input     [7:0]     out_port;

input     [3:0]     baud;

input               eight, p_en, ohel;

input               rx;

output              tx;

output    [7:0]     UART_DS;

output              UART_INT;

wire                w_tx_rdy;

wire                w_rx_rdy;

wire                w_ped_tx;

wire                w_ped_rx;

wire                w_perr;

wire                w_ferr;

wire                w_ovf;

wire      [18:0]    k;

wire      [7:0]     w_data_to_TB;

wire      [7:0]     w_status;

//***********************************

//   Baud

//***********************************

Baud_Dec

baudDec(

.baud(baud),

.baud_count(k)

);

//***********************************

//   Tx Engine

//***********************************

Tx_Engine

transmit_engine(

.clk(clk),

.rst(rst),

.load(write),

.baud_count(k),

.data_in(out_port),

.eight(eight),

.p_en(p_en),

.ohel(ohel),

.txrdy(w_tx_rdy),

.tx(tx)

);

//***********************************

//   Rx Engine

//***********************************

Rx_Engine

receive_engine(

.clk(clk),

.rst(rst),

.rx(rx),

.k(k),

.eight(eight),

.p_en(p_en),

.ohel(ohel),

.read(read[0]),

.rx_rdy(w_rx_rdy),

.parity_err(w_perr),

.frame_err(w_ferr),

.overflow(w_ovf),

.data_to_TB(w_data_to_TB)

);

assign w_status = {3'b0, w_ovf, w_ferr, w_perr, w_tx_rdy, w_rx_rdy};

assign UART_DS = (read[1]) ? w_status     :

(read[0]) ? w_data_to_TB :

8'b0;

//***********************************

//   PED Tx

//   PED Rx

//***********************************

PED

tx_ped(

.clk(clk),

.rst(rst),

.ped_in(w_tx_rdy),

.ped_out(w_ped_tx)

),

rx_ped(

.clk(clk),

.rst(rst),

.ped_in(w_rx_rdy),

.ped_out(w_ped_rx)

);

assign UART_INT = w_ped_tx | w_ped_rx;  // wire to SR Flop

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  SR_Flop.v                                 //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module SR_Flop(clk, rst, S, R, Q);

input     clk, rst;

input     S, R;

output    Q;

reg       Q;

always @(posedge clk, posedge rst)

if(rst)

Q <= 1'b0;

else if(S)

Q <= 1'b1;

else if(R)

Q <= 1'b0;

else

Q <= Q;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Baud_Dec.v                                //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Baud_Dec(baud, baud_count);

input     [3:0]     baud;

output    [18:0]    baud_count;

reg       [18:0]    baud_count;

// Baud Rate

// Calculation (1/baud rate) / (1/100MHz) - 1

always @(*)

case(baud)

4'b0000 : baud_count <= 333333 - 1;    // Baud rate 300

4'b0001 : baud_count <= 83333  - 1;    // Baud rate 1200

4'b0010 : baud_count <= 41667  - 1;    // Baud rate 2400

4'b0011 : baud_count <= 20833  - 1;    // Baud rate 4800

4'b0100 : baud_count <= 10417  - 1;    // Baud rate 9600

4'b0101 : baud_count <= 5208   - 1;    // Baud rate 19200

4'b0110 : baud_count <= 2604   - 1;    // Baud rate 38400

4'b0111 : baud_count <= 1736   - 1;    // Baud rate 57600

4'b1000 : baud_count <= 868    - 1;    // Baud rate 115200

4'b1001 : baud_count <= 434    - 1;    // Baud rate 230400

4'b1010 : baud_count <= 217    - 1;    // Baud rate 460800

4'b1011 : baud_count <= 109    - 1;    // Baud rate 921600

default : baud_count <= 333333 - 1;    // Default rate 300

endcase

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Tx_Engine.v                               //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Tx_Engine(clk, rst, load, baud_count, data_in,

eight, p_en, ohel, txrdy, tx);

input               clk, rst, load;

input     [18:0]    baud_count;

input     [7:0]     data_in;

input               eight, p_en, ohel;

output              txrdy;

output              tx;

reg                 w_load_d1;

wire                w_done;

wire                w_do_it;

wire                w_btu;

wire      [7:0]     w_load_data;

wire                w_bit10, w_bit9;

//***********************************

//   SR Flip Flop TxRdy

//***********************************

SR_Flop_Txrdy

txrdy_sr_flop(

.clk(clk),

.rst(rst),

.S(w_done),

.R(load),

.Q(txrdy)

);

//***********************************

//   SR Flip Flop DoIt

//***********************************

Tx_SR_Flop

doit_sr_flop(

.clk(clk),

.rst(rst),

.S(w_load_d1),

.R(w_done),

.Q(w_do_it)

);

//***********************************

// 8-bit Loadable Register

//***********************************

Tx_Load_Reg

tx_ld_reg(

.clk(clk),

.rst(rst),

.ld(load),

.D(data_in),

.Q(w_load_data)

);

//***********************************

// Parity Decoder

//***********************************

Tx_Parity_Gen_Dec

tx_p_gen(

.load_data(w_load_data),

.eight(eight),

.p_en(p_en),

.ohel(ohel),

.bit10(w_bit10),

.bit9(w_bit9)

);

//***********************************

//   Register

//***********************************

always @(posedge clk, posedge rst)

if(rst)

w_load_d1 <= 1'b0;

else

w_load_d1 <= load;

//***********************************

//   Shift Register

//***********************************

Tx_shift_register

tx_shift_reg(

.clk(clk),

.rst(rst),

.data_in({w_bit10, w_bit9, w_load_data[6:0], 1'b0, 1'b1}),

.load(w_load_d1),

.shift(w_btu),

.sdo(tx)

);

//***********************************

//   Bit Time Counter

//***********************************

Tx_bit_time_counter

tx_btc(

.clk(clk),

.rst(rst),

.baud_count(baud_count),

.sel({w_do_it, w_btu}),

.btu(w_btu)

);

//***********************************

//   Bit Counter

//***********************************

Tx_bit_counter

tx_bc(

.clk(clk),

.rst(rst),

.sel({w_do_it, w_btu}),

.done(w_done)

);

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Rx_Engine.v                               //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Rx_Engine(clk, rst, rx, k, eight, p_en, ohel, read,

rx_rdy, parity_err, frame_err, overflow, data_to_TB);

input               clk, rst;

input               rx;

input     [18:0]    k;

input               eight, p_en, ohel;

input               read;

output              rx_rdy;

output              parity_err;

output              frame_err;

output              overflow;

output    [7:0]     data_to_TB;

wire                w_start;

wire                w_btu;

wire                w_done;

//***********************************

//   Rx Engine Control

//***********************************

Rx_Engine_Control

control_rx(

.clk(clk),

.rst(rst),

.rx(rx),

.k(k),

.eight(eight),

.p_en(p_en),

.start(w_start),

.btu(w_btu),

.done(w_done)

);

//***********************************

//   Rx Engine Datapath

//***********************************

Rx_DataPath

datapath_rx(

.clk(clk),

.rst(rst),

.btu(w_btu),

.start(w_start),

.rx(rx),

.eight(eight),

.p_en(p_en),

.even(~ohel),

.read(read),

.done(w_done),

.rx_rdy(rx_rdy),

.parity_err(parity_err),

.frame_err(frame_err),

.overflow(overflow),

.data_to_TB(data_to_TB)

);

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  PED.v                                     //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module PED(clk, rst, ped_in, ped_out);

input     clk, rst;

input     ped_in;

output    ped_out;

reg       q1, q2;

always @(posedge clk, posedge rst)

if(rst)

{q1, q2} <= 2'b0;

else

{q1,q2} <= {ped_in, q1};

//***********************************

//   Assign Statement

//   Modeling combinational logic

//***********************************

assign ped_out = ~q2 & q1;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  SR_Flop_Txrdy.v                           //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module SR_Flop_Txrdy(clk, rst, S, R, Q);

input     clk, rst;

input     S, R;

output    Q;

reg       Q;

always @(posedge clk, posedge rst)

if(rst)

Q <= 1'b1;

else if(S)

Q <= 1'b1;

else if(R)

Q <= 1'b0;

else

Q <= Q;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Tx_Load_Reg.v                             //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Tx_Load_Reg(clk, rst, ld, D, Q);

input               clk, rst, ld;

input     [7:0]     D;

output    [7:0]     Q;

reg       [7:0]     Q;

always @(posedge clk, posedge rst)

if(rst)

begin

Q <= 8'b0;

end

else if(ld)

begin

Q <= D;

end

else

begin

Q <= Q;

end

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Tx_Parity_Gen_Dec.v                       //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

//*******************************************************//

module Tx_Parity_Gen_Dec(load_data, eight, p_en, ohel, bit10, bit9);

input     [7:0]     load_data;

input               eight, p_en, ohel;

output              bit10, bit9;

reg                 EP, OP;

reg                 bit10, bit9;

//***********************************

//   y = f(x)

//   Function is used to determine

//   Odd/Even parity bits

//***********************************

always @ (*)

begin

EP   <= (eight) ?   ^load_data[7:0]   :   ^load_data[6:0];

OP   <= (eight) ? ~(^load_data[7:0])  : ~(^load_data[6:0]);

end

always @(*)

case({eight, p_en, ohel})

3'b000 : {bit10, bit9}  <=  2'b11;                // 7N1

3'b001 : {bit10, bit9}  <=  2'b11;                // 7N1

3'b010 : {bit10, bit9}  <= {1'b1, EP};            // 7E1

3'b011 : {bit10, bit9}  <= {1'b1, OP};            // 7O1

3'b100 : {bit10, bit9}  <= {1'b1, load_data[7]};  // 8N1

3'b101 : {bit10, bit9}  <= {1'b1, load_data[7]};  // 8N1

3'b110 : {bit10, bit9}  <= {EP,   load_data[7]};  // 8E1

3'b111 : {bit10, bit9}  <= {OP,   load_data[7]};  // 8O1

endcase

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Tx_shift_register .v                      //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Tx_shift_register(clk, rst, data_in, load, shift, sdo);

input               clk, rst;

input     [10:0]    data_in;

input               load;

input               shift;

output              sdo;

reg       [10:0]    shift_reg;

always @(posedge clk, posedge rst)

if(rst)

shift_reg <= 11'b11111_111111;

else if(load)

shift_reg <= data_in;

else if(shift)

shift_reg <= {1'b1, shift_reg[10:1]};

else

shift_reg <= shift_reg;

assign sdo = shift_reg[0];

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Tx_bit_time_counter.v                     //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

//*******************************************************//

module Tx_bit_time_counter(clk, rst, baud_count, sel, btu);

input               clk, rst;

input     [18:0]    baud_count;

input     [1:0]     sel;      // concat 1-bit do_it & btu wires

output              btu;

reg       [18:0]    mux_out;

reg       [18:0]    bit_time_count;

// MUX Selector

always @(*)

case(sel)

2'b00     :    mux_out <= 19'b0;

2'b01     :    mux_out <= 19'b0;

2'b10     :    mux_out <= bit_time_count + 19'b1;

2'b11     :    mux_out <= 19'b0;

default   :    mux_out <= 19'b0;

endcase

// 19 bit Register

always @(posedge clk, posedge rst)

if(rst)

bit_time_count <= 19'b0;

else

bit_time_count <= mux_out;

// Comparator

assign btu = (bit_time_count == baud_count) ? 1'b1 : 1'b0;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Tx_bit_counter.v                          //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Tx_bit_counter(clk, rst, sel, done);

input               clk, rst;

input     [1:0]     sel;

output              done;

reg       [3:0]     mux_out;

reg       [3:0]     bit_count;

// MUX Select

always @(*)

case(sel)

2'b00 : mux_out <= 4'b0;

2'b01 : mux_out <= 4'b0;

2'b10 : mux_out <= bit_count;

2'b11 : mux_out <= bit_count + 4'b1;

endcase

// 4-bit Register

always @(posedge clk, posedge rst)

if(rst)

bit_count <= 4'b0;

else

bit_count <= mux_out;

assign done = (bit_count == 11) ? 1'b1 : 1'b0;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Rx_Engine_Control.v                       //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Rx_Engine_Control(clk, rst, rx, k, eight, p_en,

start, btu, done);

input               clk, rst;

input               rx;

input     [18:0]    k;

input               eight;

input               p_en;

output              start;

output              btu;

output              done;

wire               w_do_it;

//***********************************

//   Rx FSM

//***********************************

Rx_FSM

rx_fsm(

.clk(clk),

.rst(rst),

.rx(rx),

.btu(btu),

.done(done),

.start(start),

.do_it(w_do_it)

);

//***********************************

//   Rx Bit Time Counter

//***********************************

Rx_bit_time_counter

rx_btc(

.clk(clk),

.rst(rst),

.start(start),

.k(k),

.sel({w_do_it, btu}),

.btu(btu)

);

//***********************************

//   Rx Bit Counter

//***********************************

Rx_bit_counter

rx_bc(

.clk(clk),

.rst(rst),

.sel({w_do_it, btu}),

.eight(eight),

.p_en(p_en),

.done(done)

);

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Rx_FSM.v                                  //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Rx_FSM(clk, rst, rx, btu, done, start, do_it);

input          clk, rst;

input          rx, btu, done;

output         start, do_it;

reg            start, do_it;

reg  [1:0]     CurrentState, NxtState;

reg            NxtStart, NxtDo_It;

parameter      s0 = 2'b00,

s1 = 2'b01,

s2 = 2'b10;

always @(posedge clk, posedge rst)

begin

if(rst)

{CurrentState, start, do_it} <= {s0, 2'b00};

else

{CurrentState, start, do_it} <= {NxtState, NxtStart, NxtDo_It};

end

always @(*)

begin

NxtStart = 0;

NxtDo_It = 0;

NxtState = CurrentState;

case(CurrentState)

s0:

begin

{NxtState, NxtStart, NxtDo_It} = (rx) ? {s0, 2'b00} :

{s1, 2'b11};

end

s1:

begin

{NxtState, NxtStart, NxtDo_It} =

(rx)           ? {s0, 2'b00} :

(~rx && ~btu)  ? {s1, 2'b11} :

{s2, 2'b01};

end

s2:

begin

{NxtState, NxtStart, NxtDo_It} = (~done) ? {s2, 2'b01} :

{s0, 2'b00};

end

default   : {NxtState, NxtStart, NxtDo_It} = {s0, 2'b00};

endcase

end

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Rx_bit_time_counter.v                     //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//*******************************************************//

module Rx_bit_time_counter(clk, rst, start, k, sel, btu);

input               clk, rst;

input               start;

input     [18:0]    k;

input     [1:0]     sel;      // concat 1-bit do_it & btu wires

output              btu;

reg       [18:0]    mux_out;

reg       [18:0]    bit_time_count;

wire      [18:0]    mux_baud_count_out;

// MUX Selector

always @(*)

case(sel)

2'b00     :    mux_out <= 19'b0;

2'b01     :    mux_out <= 19'b0;

2'b10     :    mux_out <= bit_time_count + 19'b1;

2'b11     :    mux_out <= 19'b0;

default   :    mux_out <= 19'b0;

endcase

// 19 bit Register

always @(posedge clk, posedge rst)

if(rst)

bit_time_count <= 19'b0;

else

bit_time_count <= mux_out;

// Comparator

assign btu = (bit_time_count == mux_baud_count_out) ? 1'b1 : 1'b0;

// Buad Mux Select

assign mux_baud_count_out = (start) ? (k >> 1) : (k);

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Rx_bit_counter.v                          //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Rx_bit_counter(clk, rst, sel, eight, p_en, done);

input               clk, rst;

input     [1:0]     sel;

input               eight, p_en;

output              done;

reg       [3:0]     mux_out;

reg       [3:0]     bit_count;

reg       [3:0]     num_bits;

// MUX Select

always @(*)

begin

case(sel)

2'b00     :    mux_out <= 4'b0;

2'b01     :    mux_out <= 4'b0;

2'b10     :    mux_out <= bit_count;

2'b11     :    mux_out <= bit_count + 4'b1;

default   :    mux_out <= 4'b0;

endcase

end

// 4-bit Register

always @(posedge clk, posedge rst)

if(rst)

bit_count <= 4'b0;

else

bit_count <= mux_out;

// f(x) used to determine the bits

always @(*)

begin

case({eight, p_en})

2'b00     :    num_bits = 4'h9; // 7N1

2'b01     :    num_bits = 4'hA; // 7E1/7O1

2'b10     :    num_bits = 4'hA; // 8N1

2'b11     :    num_bits = 4'hB; // 8O1/8E1

default   :    num_bits = 4'hA; // 7N1

endcase

end

// Comparator

assign done = (bit_count == num_bits) ? 1'b1 : 1'b0;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 2>                               //

// File name:  Rx_DataPath.v                             //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Rx_DataPath(clk, rst, btu, start, rx,

eight, p_en, even, done, read,

rx_rdy, parity_err, frame_err, overflow, data_to_TB);

input               clk, rst;

input               btu, start, rx;

input               eight, p_en;

input               even, done;

input               read;

output              rx_rdy;

output              parity_err;

output              frame_err;

output              overflow;

output    [7:0]     data_to_TB;

reg                 stop_bit_mux_out;

wire      [9:0]     w_shift_reg_out;

wire      [9:0]     w_remap;

wire                w_p_gen_mux;

wire                w_p_gen_even_mux;

wire                w_p_bit_mux;

wire                w_perr;

wire                w_ferr;

wire                w_ovf;

//***********************************

//   Rx Shift Register

//***********************************

Rx_Shift_Reg

rx_shift_reg(

.clk(clk),

.rst(rst),

.sh(btu & ~start),

.sdi(rx),

.data_out(w_shift_reg_out)

);

//***********************************

//   Rx ReMap

//***********************************

Rx_Remap

rx_remap(

.sel({eight, p_en}),

.data_in(w_shift_reg_out),

.remap(w_remap)

);

assign w_p_gen_mux = (eight) ? w_remap[7] : 1'b0;

assign w_p_gen_even_mux = (even) ? (^{w_p_gen_mux, w_remap[6:0]}) :

~(^{w_p_gen_mux, w_remap[6:0]});

assign w_p_bit_mux = (eight) ? w_remap[8] : w_remap[7];

assign w_perr = p_en & (^{w_p_gen_even_mux, w_p_bit_mux}) & done;

//***********************************

//   Stop Bit Select MUX

//***********************************

always @(*)

case({eight, p_en})

2'b00     :    stop_bit_mux_out <= w_remap[7];

2'b01     :    stop_bit_mux_out <= w_remap[8];

2'b10     :    stop_bit_mux_out <= w_remap[8];

2'b11     :    stop_bit_mux_out <= w_remap[9];

default   :    stop_bit_mux_out <= w_remap[7];

endcase

//***********************************

//   SR Rx Ready Flop

//***********************************

Rx_SR_Flop

rx_ready(

.clk(clk),

.rst(rst),

.S(done),

.R(read),

.Q(rx_rdy)

);

//***********************************

//   SR Rx Parity Error

//***********************************

Rx_SR_Flop

p_err(

.clk(clk),

.rst(rst),

.S(w_perr),

.R(read),

.Q(parity_err)

);

//***********************************

//   SR Rx Framing Error

//***********************************

Rx_SR_Flop

framing_err(

.clk(clk),

.rst(rst),

.S(done & ~stop_bit_mux_out),

.R(read),

.Q(frame_err)

);

//***********************************

//   SR Rx Overflow Error

//***********************************

Rx_SR_Flop

ovf_err(

.clk(clk),

.rst(rst),

.S(done & rx_rdy),

.R(read),

.Q(overflow)

);

//***********************************

//   Data sent to TramelBlaze

//***********************************

assign data_to_TB = (eight) ? w_remap[7:0] : {1'b0, w_remap[6:0]};

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Rx_Shift_Reg.v                            //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Rx_Shift_Reg(clk, rst, sh, sdi, data_out);

input               clk, rst;

input               sh;

input               sdi;      // Rx input

output    [9:0]     data_out;

reg       [9:0]     data_out;

always @(posedge clk, posedge rst)

if(rst)

data_out <= 10'b0;

else if(sh)

data_out <= {sdi, data_out[9:1]};

else

data_out <= data_out;

endmodule

`timescale 1ns / 1ps

//*******************************************************//

// This document contains information proprietary        //

// to the CSULB student that created the                 //

// file - any reuse without adequate approval and        //

// documentation is prohibited                           //

//                                                       //

// Class:      <CECS 460 SOC>                            //

// Project:    <Project 3>                               //

// File name:  Rx_Remap.v                                //

//                                                       //

// Created by <Jose Sotelo> on <>                        //

//                                                       //

// In submitting this file for class work at CSULB       //

// I am confirming that this is my work and the work     //

// of no one else.                                       //

//                                                       //

// In the event other code source are utilized I will    //

// document which portion of code and who is the author  //

//                                                       //

// In submitting this code I acknowledge that plagiarism //

// in student project work is subject to dismissal from  //

// the class                                             //

//*******************************************************//

module Rx_Remap(sel, data_in, remap);

input     [1:0]     sel;           // eight and p_en

input     [9:0]     data_in;       // shift reg data

output    [9:0]     remap;         // remap combo

reg       [9:0]     remap;

always @(*)

case(sel)

2'b00     :    remap <= {2'b00, data_in[9:2]};

2'b01     :    remap <= {1'b0,  data_in[9:1]};

2'b10     :    remap <= {1'b0,  data_in[9:1]};

2'b11     :    remap <= data_in;

default   :    remap <= {2'b00, data_in[9:2]};

endcase

endmodule

; Jose Sotelo

; CECS 460

; Full duplex UART and TramelBlaze Assembly

;=======================================================

;   Define Variables Alias

;=======================================================

ZERO                EQU     0000

ONE                 EQU     0001

PORT                EQU     0001

LED_PORT            EQU     0005

RX_RDY              EQU     0001

TX_RDY              EQU     0002

ASCII_BS            EQU     0008

ASCII_TAB           EQU     0009

ASCII_LF            EQU     000A

ASCII_CR            EQU     000D

ASCII_SLASH         EQU     002F

ASCII_EQUALS        EQU     003D

ASCII_ASTERISK      EQU     002A

ASCII_AT            EQU     0040

ASCII_DOT           EQU     002E

ASCII_SPACE         EQU     0020

ASCII_GREATER       EQU     003E

ASCII_COLON         EQU     003A

ASCII_B_SLASH       EQU     005C

ASCII_ZERO          EQU     0030

ASCII_ONE           EQU     0031

ASCII_TWO           EQU     0032

ASCII_THREE         EQU     0033

ASCII_FOUR          EQU     0034

ASCII_FIVE          EQU     0035

ASCII_SIX           EQU     0036

ASCII_SEVEN         EQU     0037

ASCII_EIGHT         EQU     0038

ASCII_NINE          EQU     0039

ASCII_A             EQU     0041

ASCII_B             EQU     0042

ASCII_C             EQU     0043

ASCII_D             EQU     0044

ASCII_E             EQU     0045

ASCII_F             EQU     0046

ASCII_G             EQU     0047

ASCII_H             EQU     0048

ASCII_I             EQU     0049

ASCII_J             EQU     004A

ASCII_K             EQU     004B

ASCII_L             EQU     004C

ASCII_M             EQU     004D

ASCII_N             EQU     004E

ASCII_O             EQU     004F

ASCII_P             EQU     0050

ASCII_Q             EQU     0051

ASCII_R             EQU     0052

ASCII_S             EQU     0053

ASCII_T             EQU     0054

ASCII_U             EQU     0055

ASCII_V             EQU     0056

ASCII_W             EQU     0057

ASCII_X             EQU     0058

ASCII_Y             EQU     0059

ASCII_Z             EQU     005A

;=======================================================

;   Register Alias

;=======================================================

TEMP_REG            EQU     R0

POINTER             EQU     R1

COUNTER             EQU     R2

COUNT               EQU     R3

UART_STATUS         EQU     R4

CURRENT_CASE        EQU     R5

CHAR_COUNT          EQU     R6

STORE_DATA          EQU     R7

;=======================================================

;   Start

;   Startup Code Initialization

;   only executed at startup (reset)

;=======================================================

START

LOAD    TEMP_REG,     ZERO

LOAD    POINTER,      ZERO

LOAD    COUNTER,      ZERO

LOAD    COUNT,        ZERO

LOAD    UART_STATUS,  ZERO

LOAD    CURRENT_CASE, ONE

CALL    BANNER

CALL    PROMPT_USER

CALL    TOWN

CALL    BACKSPACE

CALL    CRLF

ENINT

JUMP    MAIN

;=======================================================

;   Banner Subroutine

;   Banner is executed on startup

;   /////////////////////////////////

;       CSULB CECS 460

;       Jose Sotelo

;   /////////////////////////////////

;=======================================================

BANNER

LOAD    TEMP_REG,   ASCII_SLASH

ADD_SLASH   STORE   TEMP_REG,   COUNTER

ADD     COUNTER,    0001

COMP    COUNTER,    0028

JUMPC   ADD_SLASH

LOAD    TEMP_REG,   ASCII_CR

STORE   TEMP_REG,   0029

LOAD    TEMP_REG,   ASCII_LF

STORE   TEMP_REG,   002A

LOAD    TEMP_REG,   ASCII_TAB

STORE   TEMP_REG,   002B

LOAD    TEMP_REG,   ASCII_C

STORE   TEMP_REG,   002C

LOAD    TEMP_REG,   ASCII_S

STORE   TEMP_REG,   002D

LOAD    TEMP_REG,   ASCII_U

STORE   TEMP_REG,   002E

LOAD    TEMP_REG,   ASCII_L

STORE   TEMP_REG,   002F

LOAD    TEMP_REG,   ASCII_B

STORE   TEMP_REG,   0030

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   0031

LOAD    TEMP_REG,   ASCII_C

STORE   TEMP_REG,   0032

LOAD    TEMP_REG,   ASCII_E

STORE   TEMP_REG,   0033

LOAD    TEMP_REG,   ASCII_C

STORE   TEMP_REG,   0034

LOAD    TEMP_REG,   ASCII_S

STORE   TEMP_REG,   0035

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   0036

LOAD    TEMP_REG,   ASCII_FOUR

STORE   TEMP_REG,   0037

LOAD    TEMP_REG,   ASCII_SIX

STORE   TEMP_REG,   0038

LOAD    TEMP_REG,   ASCII_ZERO

STORE   TEMP_REG,   0039

LOAD    TEMP_REG,   ASCII_CR

STORE   TEMP_REG,   003A

LOAD    TEMP_REG,   ASCII_LF

STORE   TEMP_REG,   003B

LOAD    TEMP_REG,   ASCII_TAB

STORE   TEMP_REG,   003C

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   003D

LOAD    TEMP_REG,   ASCII_J

STORE   TEMP_REG,   003E

LOAD    TEMP_REG,   ASCII_O

STORE   TEMP_REG,   003F

LOAD    TEMP_REG,   ASCII_S

STORE   TEMP_REG,   0040

LOAD    TEMP_REG,   ASCII_E

STORE   TEMP_REG,   0041

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   0042

LOAD    TEMP_REG,   ASCII_S

STORE   TEMP_REG,   0043

LOAD    TEMP_REG,   ASCII_O

STORE   TEMP_REG,   0044

LOAD    TEMP_REG,   ASCII_T

STORE   TEMP_REG,   0045

LOAD    TEMP_REG,   ASCII_E

STORE   TEMP_REG,   0046

LOAD    TEMP_REG,   ASCII_L

STORE   TEMP_REG,   0047

LOAD    TEMP_REG,   ASCII_O

STORE   TEMP_REG,   0048

LOAD    TEMP_REG,   ASCII_CR

STORE   TEMP_REG,   0049

LOAD    TEMP_REG,   ASCII_LF

STORE   TEMP_REG,   004A

LOAD    TEMP_REG,   ASCII_TAB

STORE   TEMP_REG,   004B

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   004C

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   004D

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   004E

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   004F

LOAD    TEMP_REG,   ASCII_U

STORE   TEMP_REG,   0050

LOAD    TEMP_REG,   ASCII_A

STORE   TEMP_REG,   0051

LOAD    TEMP_REG,   ASCII_R

STORE   TEMP_REG,   0052

LOAD    TEMP_REG,   ASCII_T

STORE   TEMP_REG,   0053

LOAD    TEMP_REG,   ASCII_CR

STORE   TEMP_REG,   0054

LOAD    TEMP_REG,   ASCII_LF

STORE   TEMP_REG,   0055

LOAD    TEMP_REG,   ASCII_SLASH

LOAD    COUNT,      0056

LAST        STORE   TEMP_REG,   COUNT

ADD     COUNT,      0001

COMP    COUNT,      007E

JUMPC   LAST

LOAD    TEMP_REG,   ASCII_CR

STORE   TEMP_REG,   007F

LOAD    TEMP_REG,   ASCII_LF

STORE   TEMP_REG,   0080

;=======================================================

;   Prompt User Subroutine

;   Allows the user to type

;   C:\User>

;=======================================================

PROMPT_USER

LOAD    TEMP_REG,   ASCII_C

STORE   TEMP_REG,   0081

LOAD    TEMP_REG,   ASCII_COLON

STORE   TEMP_REG,   0082

LOAD    TEMP_REG,   ASCII_B_SLASH

STORE   TEMP_REG,   0083

LOAD    TEMP_REG,   ASCII_U

STORE   TEMP_REG,   0084

LOAD    TEMP_REG,   ASCII_S

STORE   TEMP_REG,   0085

LOAD    TEMP_REG,   ASCII_E

STORE   TEMP_REG,   0086

LOAD    TEMP_REG,   ASCII_R

STORE   TEMP_REG,   0087

LOAD    TEMP_REG,   ASCII_GREATER

STORE   TEMP_REG,   0088

;=======================================================

;   TOWN Subroutine

;   Prints the designer's hometown

;   Hometown: Boyle Heights

;=======================================================

TOWN

LOAD    TEMP_REG,   ASCII_H

STORE   TEMP_REG,   0089

LOAD    TEMP_REG,   ASCII_O

STORE   TEMP_REG,   008A

LOAD    TEMP_REG,   ASCII_M

STORE   TEMP_REG,   008B

LOAD    TEMP_REG,   ASCII_E

STORE   TEMP_REG,   008C

LOAD    TEMP_REG,   ASCII_T

STORE   TEMP_REG,   008D

LOAD    TEMP_REG,   ASCII_O

STORE   TEMP_REG,   008E

LOAD    TEMP_REG,   ASCII_W

STORE   TEMP_REG,   008F

LOAD    TEMP_REG,   ASCII_N

STORE   TEMP_REG,   0090

LOAD    TEMP_REG,   ASCII_COLON

STORE   TEMP_REG,   0091

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   0092

LOAD    TEMP_REG,   ASCII_B

STORE   TEMP_REG,   0093

LOAD    TEMP_REG,   ASCII_O

STORE   TEMP_REG,   0094

LOAD    TEMP_REG,   ASCII_Y

STORE   TEMP_REG,   0095

LOAD    TEMP_REG,   ASCII_L

STORE   TEMP_REG,   0096

LOAD    TEMP_REG,   ASCII_E

STORE   TEMP_REG,   0097

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   0098

LOAD    TEMP_REG,   ASCII_H

STORE   TEMP_REG,   0099

LOAD    TEMP_REG,   ASCII_E

STORE   TEMP_REG,   009A

LOAD    TEMP_REG,   ASCII_I

STORE   TEMP_REG,   009B

LOAD    TEMP_REG,   ASCII_G

STORE   TEMP_REG,   009C

LOAD    TEMP_REG,   ASCII_H

STORE   TEMP_REG,   009D

LOAD    TEMP_REG,   ASCII_T

STORE   TEMP_REG,   009E

LOAD    TEMP_REG,   ASCII_S

STORE   TEMP_REG,   009F

LOAD    TEMP_REG,   ASCII_CR

STORE   TEMP_REG,   00A0

LOAD    TEMP_REG,   ASCII_LF

STORE   TEMP_REG,   00A1

;=======================================================

;   Backspace

;   Allows the user to move the display cursor one

;   position backwards and deletes the character at

;   that position

;=======================================================

BACKSPACE

LOAD    TEMP_REG,   ASCII_BS

STORE   TEMP_REG,   00A2

LOAD    TEMP_REG,   ASCII_SPACE

STORE   TEMP_REG,   00A3

LOAD    TEMP_REG,   ASCII_BS

STORE   TEMP_REG,   00A4

RETURN

;=======================================================

;   Carriage Return and line feed

;   They're used to note the termination of a line

;=======================================================

CRLF

LOAD    TEMP_REG,   ASCII_CR

STORE   TEMP_REG,   00A5

LOAD    TEMP_REG,   ASCII_LF

STORE   TEMP_REG,   00A6

RETURN

;=======================================================

;   TX Subroutine

;=======================================================

TX

COMP    CURRENT_CASE, ZERO

RETURNZ

FETCH   TEMP_REG,   POINTER

OUTPUT  TEMP_REG,   ZERO

ADD     POINTER,    ONE

COMP    CURRENT_CASE, 0001

JUMPZ   SHOW_BANNER

COMP    CURRENT_CASE, 0002

JUMPZ   SHOW_PROMPT_USER

COMP    CURRENT_CASE, 0003

JUMPZ   SHOW_TOWN

COMP    CURRENT_CASE, 0004

JUMPZ   SHOW_BACKSPACE

COMP    CURRENT_CASE, 0005

JUMPZ   SHOW_CRLF

COMP    CURRENT_CASE, 0006

JUMPZ   SHOW_COUNT

RETURN

SHOW_BANNER

COMP    POINTER,      0081

RETURNC

LOAD    CURRENT_CASE, 0002

RETURN

SHOW_PROMPT_USER

COMP    POINTER,      0089

RETURNC

LOAD    CURRENT_CASE, 0000

RETURN

SHOW_TOWN

COMP    POINTER,      00A2

RETURNC

LOAD    POINTER,      0081

LOAD    CURRENT_CASE, 0002

RETURN

SHOW_BACKSPACE

COMP    POINTER,      00A5

RETURNC

LOAD    CURRENT_CASE, 0000

RETURN

SHOW_CRLF

COMP    POINTER,      00A7

RETURNC

LOAD    POINTER,      0081

LOAD    CURRENT_CASE, 0002

RETURN

SHOW_COUNT

COMP    POINTER,      00A9

RETURNC

LOAD    POINTER,      00A5

LOAD    CURRENT_CASE, 0005

RETURN

;=======================================================

;   RX Subroutine

;=======================================================

RX

COMP    CURRENT_CASE,   ZERO

RETURNNZ

INPUT   STORE_DATA,     ZERO

COMP    STORE_DATA,     ZERO

RETURNZ

COMP    STORE_DATA,     ASCII_ASTERISK

JUMPZ   RECEIVE_TOWN

COMP    STORE_DATA,     ASCII_BS

COMP    STORE_DATA,     ASCII_CR

JUMPZ   RECEIVE_CRLF

COMP    STORE_DATA,     ASCII_AT

JUMPZ   RECIEVE_AT

;used to display char count

ADD     CHAR_COUNT,     0001

OUTPUT  STORE_DATA,     ZERO

COMP    CHAR_COUNT,     0029

JUMPZ   RECEIVE_CRLF

RETURN

RECEIVE_TOWN

LOAD    CURRENT_CASE, 0003

LOAD    POINTER,      0089

LOAD    TEMP_REG,     0000

OUTPUT  TEMP_REG,     0000

LOAD    CHAR_COUNT,   0000

RETURN

RECEIVE_BACKSPACE

COMP    CHAR_COUNT,   0000

RETURNZ

LOAD    CURRENT_CASE, 0004

LOAD    POINTER,      00A2

LOAD    TEMP_REG,     0000

OUTPUT  TEMP_REG,     0000

SUB     CHAR_COUNT,   0001

RETURN

RECEIVE_CRLF

LOAD    CURRENT_CASE, 0005

LOAD    POINTER,      00A5

LOAD    TEMP_REG,     0000

OUTPUT  TEMP_REG,     0000

LOAD    CHAR_COUNT,   0000

RETURN

RECIEVE_AT

CALL    BinToASC

LOAD    CURRENT_CASE, 0006

LOAD    Pointer,      00A7

LOAD    TEMP_REG,     0000

OUTPUT  TEMP_REG,     0000

LOAD    CHAR_COUNT,   0000

RETURN

;=======================================================

;   BIN_TO_ASCII

;=======================================================

BinToASC

LOAD    RE, CHAR_COUNT

LOAD    RD, 000A

CALL    FINDIT

ADD     RB, 0030

STORE   RB, 00A7

ADD     RE, 0030

STORE   RE, 00A8

RETURN

;=======================================================

;   FInd It Routine

;=======================================================

FINDIT

LOAD    RB, ZERO

REPEAT  SUB     RE, RD

JUMPC   FOUNDIT

ADD     RB, 0001

JUMP    REPEAT

FOUNDIT

ADD     RE, RD

RETURN

;=======================================================

;   ISR

;=======================================================

ADDRESS 0300

ISR

INPUT   UART_STATUS,    PORT

AND     UART_STATUS,    0003

COMP    UART_STATUS,    0003

JUMPZ   BOTH_RX_TX

COMP    UART_STATUS,    TX_RDY

CALLZ   TX

COMP    UART_STATUS,    RX_RDY

CALLZ   RX

RETEN

BOTH_RX_TX

CALL    TX

CALL    RX

RETEN

;=======================================================

;   Main Loop

;   main loop is where processor spends most of its time

;=======================================================

MAIN

; update LEDs

JUMP    MAIN

;=======================================================

;   ISR vectored through 0FFE

;=======================================================

ADDRESS 0FFE

ENDIT

JUMP ISR

END

Appendix A: Key Terms

Start Bit

The UART data transmission line is normally held at a high voltage level when it’s not transmitting data. To begin the transfer of data, the Tx pin pulls the transmission line from high too low for one clock cycle. When the receiving UART detects the high to low voltage transition, it begins reading the bits in the data frame at the frequency of the baud rate selected.

Data Frame/Packet

The data frame contains the actual data being transferred. It can be 5 to 8 bits long if a parity bit is used. If no parity bit is used, the data frame can be 9 bits long. In certain cases, the data sent can begin with the least significant bit first.

Parity

A parity bit is form of error checking that describes the evenness or oddness of a number. The parity bit is a way for the receiving UART to tell if any data has changed during transmission. If the parity bit is a 1 (odd parity), then there are an odd number of 1 bits in the data frame. If the parity bit is a 0 (even parity), then there are an even number of 1 bits in the data frame.

Stop Bit

To signal the end of the data transmission, the sending UART drives the data transmission line from a low voltage to a high voltage.

Baud Rate

The baud rate identifies the frequency the data is being transmitted at. (Bits per second)

Bit time

Bit time is the amount of time data bits are held on the wire.

Figure : Illustration of Data Transmission